Chapter 2 the water shed

- Definition:

A watershed (also called a drainage basin or catchment area) is the geographic area that drains water to a common point. A watershed is a land area that channels rainfall and snowmelt into creeks, streams, and rivers, eventually leading to a common outlet such as a larger river, lake, or ocean.

- Characteristics of a Watershed:

- Drainage Divide:

  - The high land or ridge that separates one watershed from another.

- Outflow Point:

  - The location where water exits the watershed, such as a river mouth or lake.

- Area:

  - Size can vary from a few square kilometers to thousands, depending on the watershed.

- Slope:

  - Influences how quickly water flows; steeper slopes lead to faster runoff.

- Soil Type:

  - Affects infiltration and runoff. Sandy soils absorb more water; clayey soils lead to more surface runoff.

- Land Use and Vegetation:

  - Forested areas reduce runoff and erosion; urban areas increase it due to impermeable surfaces.

- Hydrological Cycle:

  - Includes processes like precipitation, infiltration, surface runoff, and evapotranspiration.

- Human Impact:

  - Dams, agriculture, deforestation, and urbanization can alter natural flow and water quality.

#### Key Elements:

- Crest (divide): The highest boundary that separates two watersheds.

- Springs: Points where groundwater emerges and starts forming streams.

- Streams and rivers: These collect water from higher elevations and flow toward lower ones.

- Outlet: The point where all the water from the watershed exits (e.g., into a lake or sea).

- Hypsometric Curve

- The hypsometric curve describes how elevation is distributed within a watershed.

Crête (ligne de partage des eaux)

/\

/  \           (Pluie)

Montagnes  /    \       ☁️☁️☁️🌧🌧🌧

/      \       /       \

/        \     /         \

/          \   /           \

/            \ /             \

/              |               \

Sources        Ruisseaux         Sources

\              |               /

\             |              /

\            |             /

\         Rivière principale

\          |          /

\         |         /

\       |       /

\     |     /

\   |   /

Exutoire (lac, mer, etc.)

- 1. Hypsometric Curve

- A hypsometric curve (or hypsographic curve) represents the distribution of elevation in a given area, such as a drainage basin.

- It plots relative elevation (height from 0 to 1) on the Y-axis against relative area (portion of total area) on the X-axis.

- Purpose: It shows how much land lies at different elevations. It’s useful for understanding erosional stages of a landscape (youthful, mature, old).

- A convex curve implies a youthful stage, while a concave curve suggests mature or old topography.

- 2. Equivalent Rectangle

- In geomorphological studies, the equivalent rectangle refers to a rectangle that has the same area as the shape being studied (like a watershed or drainage basin) but may have different dimensions.

- It’s used to simplify irregular shapes to a rectangle for easier calculation of morphometric indices like compactness or shape index.

- 3. Compactness Index (Compactness Coefficient)

- A measure of how compact or circular a shape is.

- Common formula:

C= P/2(πA)0.5

Where:

  - P = Perimeter of the area

  - A = Area

- Interpretation: A compactness index close to 1 means the shape is close to a perfect circle (most compact); higher values suggest more elongation or irregularity.

- 4. Shape Index (Form Factor or Elongation Ratio)

- Quantifies the elongation of a shape.

- There are different versions, like:

  - Form Factor:

Ff = A/L2

  A = Area

  L= Maximum length

- Hypsometric Curve Example

Example Basin Data:

Parameter	Value

Area (A)	100 km²

Perimeter (P)	50 km

Max Length (L)	20 km

Elevation max	500 m

Elevation min	100 m

Hypsometric Data	Relative elevation & area

Let’s say we have the following elevation distribution:

| Relative Area (x) | Relative Elevation (y) |
| --- | --- |
| 0.0 | 0.0 |
| 0.2 | 0.5 |
| 0.4 | 0.7 |
| 0.6 | 0.85 |
| 0.8 | 0.95 |
| 1.0 | 1.0 |

- You can plot this on a graph (x-axis = relative area, y-axis = relative elevation).

- This curve is convex upward, suggesting a youthful basin with less erosion.

- Equivalent Rectangle

To create an equivalent rectangle:

- Area (A) = 100 km²

- Assume you choose a rectangle width = 10 km, then:

Length= 100/10 = 10 KM

So, your equivalent rectangle is 10 km x 10 km, maintaining the same area as the real basin.

- 3. Compactness Index

Use the formula:

C= 50/35.44= 1.41

Since 1 is a perfect circle, a value of 1.41 suggests this basin is moderately compact but not circular.

### 4. Shape Index / Form Factor / Elongation Ratio

#### a. Form Factor:

Ff = A/L2 = 100/400= 0.25

- A low form factor (<0.5) means the basin is elongated.

#### b. Elongation Ratio:

Re​= 2(100/3.14)0.5 /20 = 0.56

- A value < 1 indicates elongation; closer to 1 means more circular.

- Interpretation:

| Metric | Value | Interpretation |
| --- | --- | --- |
| Hypsometric Curve | Convex | Youthful basin |
| Compactness Index | 1.41 | Moderately compact (not circular) |
| Form Factor | 0.25 | Elongated shape |
| Elongation Ratio | 0.56 | Elongated basin |